HOT EMISSION MODEL FOR MOBILE SOURCES: APPLICATION TO THE METROPOLITAN REGION OF THE CITY OF SANTIAGO, CHILE

## Roberto M. Corvalán, Mauricio Osses

Mechanical Engineering Department, University of Chile, Santiago, Chile

Cristian M. Urrutia

National Environmental Center, CENMA, Santiago - Chile

IMPLICATIONS

The impact of road transport on air quality in urban areas makes necessary accurate emission estimation. This paper presents a methodology to calculate real - world hot emissions from different vehicle categories under both spatial and temporal distributions. Examples of application of the methodology to the road network of the urban area at the city of Santiago, Chile, shows its power and resolution when applied by decision makers and researchers in order to reliably simulate micro-scale impact assessment analysis and prediction of future environmental situations.

ABOUT THE AUTHORS

Dr. Roberto M. Corvalán is an Assistant Professor of the Mechanical Engineering Department, of the University of Chile. E-mail: rocorval@cec.uchile.cl. Dr. Mauricio Osses holds the position of Assistant Profesor at the Mechanical Engineering Department, of the University of Chile. E-mail: maosses@cec.uchile.cl. Cristian M. Urrutia currently works on various environmental projects with members of the academic staff of the Department of Mechanical Engineering of University of Chile and his permanent position is on the professional staff of the National Environmental Center. Email: currutia@cec.uchile.cl.

## ABSTRACT

Depending on the final application, several methodologies for traffic emission estimation have been developed. Emission estimation based on total miles traveled or other average factors could be a good enough approach only for extended areas, like national or worldwide areas.   For road emission control and strategies design, micro-scale analysis based on real world emission estimations is often required, which involves actual driving behavior and emission factors of the local vehicle fleet under study.

This paper reports on a micro-scale model for hot road emissions and its application to the metropolitan region of the city of Santiago, Chile. The methodology considers the street-by-street hot emission estimation with its temporal and spatial distribution. The input data comes from experimental emission factors based on local driving patterns and traffic surveys of traffic flows for different vehicle categories.

The methodology developed is able to estimate hourly hot road CO; THC; PM and NOx emission for pre-defined day types and vehicle categories.

INTRODUCTION

Total mobile sources can be calculated as the sum of three specific terms1-2: hot; cold start and evaporative emissions. Hot emission corresponds to the exhaust pipe emissions under normal operational conditions, cold starts represent additional emissions due to lower operating engine temperature and evaporative emissions denote the fuel evaporation during travel under daily temperature fluctuations (diurnal, hot soak and running losses). Hot emissions are normally calculated multiplying a speed dependent emission factor corresponding to the pollutant and vehicle category analyzed by a term representing the activity level of the vehicle category 3-4-5. Emission factor represents the emission rate per unit of activity, normally expressed in g/km, and determined experimentally from chassis dynamometer transient tests, applying driving cycles representing local driving patterns 6-7-8-9. Activity level could be simply represented by total kilometers traveled by vehicle category, however this approach leads to poor accuracy that prevents any spatial or temporal emission distributions on the traffic network in an urban zone. More adequate approaches consist on traffic characterizations in terms of flow densities and vehicle category compositions. This can be done from traffic surveys, traffic models or from a combination of both10-11.

Hot vehicles emissions depend on several parameters, like traffic conditions (congestion), vehicle-operating conditions (average speed, load, trip length, driving mode, mileage), vehicle technological parameters (model and year, state of maintenance, engine type, control emission system), fuel characteristics (type, chemical composition) and topography. An emission calculating methodology must take into account as many as possible of these parameters.

In order to calculate emission from mobile sources, several emission models were developed based on different kind of input data, all of which lead to a wide range of accuracy12-13-14-15-16-17. Accuracy requirements further depend on field applications of the results, being higher when micro-scale analysis is required. Current trends are based on real-time traffic data taken from direct street observations.

In this paper a methodology for estimating hot emission from mobile sources is presented. The methodology considers input data from transport modeling and experimental data concerning driving cycles, emission factors and traffic flow characterization, in terms of vehicle category composition and driving modes. The emission model developed allows hourly street-by-street hot emission calculation of carbon monoxide (CO), nitrogen oxides (NOx) and total unburned hydrocarbons (THC). An application of the methodology for the traffic network of the Metropolitan Region of Santiago is presented.

METHODOLOGY

General Approach

The methodology developed calculates hourly hot emissions at a portion of street (arc) from each vehicle category present on a traffic network, as follows:

E h,i,j,k = Fj · Lj · EF i,k (v) · FPj ·· Cj,k					(1)

where:

Eh,i,j,k	: 	emission of pollutant i during the hour of the day h on the arc of the traffic network j from vehicle category k, expressed in g/h

Fj	:	total traffic volume at the arc j, on-peak traffic hours, expressed in veh/h

L j	:	total length of the arc j, in km

EF i,k (v)	:	emission factor of pollutant i and vehicle category k, as a function of average speed v, in g/km-veh

FPj	:	hourly traffic flow profile at the arc j, for different day types

Cj,k	:	fraction of the total traffic volume corresponding to the vehicle category k on the arc j

Total traffic volume, Fj, corresponds to output data from a transportation model so called ESTRAUS 18-19, with an urban road network comprising 7567 arcs distributed over the greater Santiago area, which is divided into 264 zones. ESTRAUS is one of the fundamental elements of the methodology for urban transport strategic plans evaluation, developed by the Chilean Government in the middle of 80´s decade. As with most transportation models, ESTRAUS is built on four main stages: trip generation/attraction, trip distribution, modal choice/split, and trip assignment. The last three stages are solved through a simultaneous equilibrium, generating a solution that reflects a dynamic interaction among them, instead of a sequential modeling approach. This model is used for strategic planning and covers two main periods of time: morning peak hour (typically 07:30-8:30 hrs) and the inter-peak hour (typically 10:30-11:30 hrs), both timings corresponding to a weekday.

Regarding emission calculation, the main outputs from ESTRAUS are traffic volume and time needed to drive over a given arc of the urban road network. Since the ESTRAUS model is mainly used for demand analysis, traffic volume is divided only into two types: fix and variable flow. Fix flow represents public transportation (urban buses) and it is received as input by the model, but it is modified during certain stages of modeling, especially at peak-time hours. On the other hand, variable flow includes private transportation modes and it is associated to variable timing, which allows calculating the average speed while driving over a given arc. Fig 1 shows the traffic network for the metropolitan region of the city of Santiago, Chile. In order to perform emission calculations, this traffic network is complemented with an additional highway network around the metropolitan region.

Traffic flow profiles, FPj, refer to the daily flow tendencies normalized with respect to on-peak data for each day type considered. Four types of days in a week have been defined: Monday to Thursday, Friday, Saturday and Sunday. The day type definition considers homogeneous traffic characteristics, like flow densities and hours when on and off-peaks periods take place.

The transportation model provides traffic volume desegregated only in two kinds of flows (fixed and variable), which involves only two aggregated vehicles types. A characterization of traffic flows by additional vehicles categories is however necessary, as will be explained in another section of this paper. The parameter Cj,k, represents the fraction of the total traffic volume corresponding to the vehicle category k on arc j.  Vehicle categories were defined taking into account vehicle type, final use, technology and emission standards. Table 1 shows vehicle categories considered for the emission model.

Emission factors, FE ik, came from experimental programs for gasoline light duty vehicles and from COPERT II21 for the rest of vehicle categories. The experimental program for the determination of emission factors for light-duty gasoline vehicles has been previously reported 6, where nine representative driving cycles (STD driving cycles) with the same number of average speeds were used. Driving cycles were obtained by means an instrumented car running predetermined circuits around the city, as will be discussed in detail in another section of this paper.

A general scheme of the above methodology is described in Fig. 2, indicating type and source of the input data.

Driving Cycles

Driving cycles are one of the most important parameters that influence hot exhaust emissions, as reported in previous researches 21-23. In order to get real-world emission estimates  real world driving cycles are required which can be obtained from modeling, travel survey data or measurements obtained by an instrumented car.

The emission model described in this paper uses driving cycles obtained by on-road measurement methodologies. A vehicle fitted with data acquisition system for instantaneous engine rpm and distance driven over a pre-defined circuit around the city, applying the floating car technique, results in speed-distance plots like the one shown in Figure 3. Further acceleration and speed histogram analysis leads to the definition of nine typical driving patterns with average speeds from 3 to 80 km/h. Figure 4 shows as an example a driving cycle with 30 km/h speed average (STD30). On Table 2, the main characteristics of the local driving cycles are presented.

Emission Factors

For light-duty gasoline vehicles, an experimental program to determine emission factors was conducted6. This program involved chassis dynamometer transient test using driving cycles described above (STD). A total of 166 vehicles constituted the sample, including catalytic and non-catalytic cars. Almost 2000 individual emission tests were performed. Equations (2) to (4) and (5) to (7), represent the emission factors obtained as a function of average speed, V, for catalytic and non-catalytic private light duty gasoline vehicles, respectively.

EF PRIVATE-CATALYST (CO) = 20.844 · V – 0.7656							(2)

EF PRIVATE-CATALYST NOx) = 6 ·10 -7 · V 3 – 9 ·10 -5 · V 2 + 0.0019 · V + 0.3154 		(3)

EF PRIVATE-CATALYST (THC) = 0.3681 · V –0.4085						(4)

EF PRIVATE-NON-CATALYST (CO) = 0.0243 · V 2 – 2.5613 · V + 76.977 				(5)

EF PRIVATE-NON-CATALYST  (NOx) = 8 ·10 -6 · V 3 – 0.0015 · V 2 + 0.0719 · V + 1.4986	(6)

EF PRIVATE-NON-CATALYST (THC) = 8.8083 · V –0.4792						(7)

Different emission standards have to be met by commercial and private cars; due to this reason differentiated emission factors were determined for commercial vehicles. Equation (8) to (10) and (11) to (13), correspond to the best fit of experimental data obtained for commercial catalytic and non-catalytic vehicles respectively.

EF COMMERCIAL-CATALYST (CO) = -8 ·10 –5 · V 2 + 0.0011 · V + 0.5633			(8)

EF COMMERCIAL-CATALYST (NOx) = 8 ·10-6 · V 3 + 0.001· V 2  + 0.03 · V + 0.4419	   	 (9)

EF COMMERCIAL-CATALYST (THC) = 0.0003 · V 2 – 0.0277 · V + 0.7856			(10)

EF COMMERCIAL-NON-CATALYST (CO) = 0.0228 · V 2 – 2.4598 · V + 79.998			(11)

EF COMMERCIAL-NON-CATALYST (NOx) = 3 ·10-5 V 3 – 0.0042 · V 2 + 0.1669 · V + 1.738	(12)

EF COMMERCIAL-NON-CATALYST (THC) = 0.0006 · V 2 – 0.0891 · V + 4.5941			(13)

Emission factors for taxis were assumed equal to emission factor corresponding to private vehicles, due to similar technical characteristics. A comparative analysis of local emission factors (eqs. 2 to 13) with respect to those proposed by other sources like COPERT and US-EPA-AP-42, was presented on previous paper of the authors6. This analysis denotes that local emission factor for private and commercial non-catalytic vehicles, forecasts higher emissions than AP-42, being more close to COPERT II. Higher speed influence  was observed on emission for local results and COPERT II in comparison with AP-42 approach, which is more evident for CO and THC. A better agreement was obtained between experimental data and COPERT II and AP-42, for catalytic vehicles. Nevertheless important discrepancies are noted on low speed ranges, where CO and THC emission predicted by models are lower than experimental data.

For diesel vehicles and motorcycles, no experimental facilities were available, thus emission factors from COPERT II24 were used, which are presented in Table 3. It is necessary to remark that PM emission factors for buses type A and D was corrected incrementing by approximately 20%  the COPERT´s proposal, considering the particulate low level in maintenance state of these buses category in local fleet. On the other hand, emission factors for trucks type A were obtained from a pondered combination of emission factors for light and medium trucks proposed by COPERT.

As an example, on Table 4 emission factors evaluated at 20 km/h, which should be considered as a representative average speed for the city, is presented and compared with emission factors from COPERT II evaluated for the same speed.

Traffic Flow Characterization

The hot emission model involves the terms FPj and C jk  (eq. 1) representing 24 hours traffic flow profiles and the flow fraction corresponding to the vehicle category k at the arc j, respectively. As was mentioned above, output data from traffic model ESTRAUS includes traffic volumes desegregated into fixed and variables flow with no distinction made for vehicles categories that must be considered for hourly emission calculations for each vehicle category present at the local vehicle fleet, according to equation (1). On the other hand, traffic model gives data only for two-day periods (on-peak and off-peak data). Temporal extrapolation of traffic profiles and their desegregation into vehicle categories was needed.

In order to estimate CkJ for field traffic, a characterization of different vehicle categories was carried out using a total of 75 manual measuring points distributed around the city. On each counting point, traffic flow volumes for 15 vehicle categories were identified through on-peak and off-peak periods (shown on Table 1). Results were expressed as a fraction of total traffic volume corresponding to each vehicle category, which was calibrated by data collected from automatic total traffic volume counting stations, some of which were placed close to the manual traffic count points. Data analysis permitted to define nine city sectors where vehicle category distributions were homogeneous with a statistical dispersion of less than 10%. On the other hand, as a simplification of the problem, vehicle category distributions were considered constant along the day due to the low variations obtained, as is shown in the example of Figure 5.   Figure 6 shows a graphical representation of the nine sectors defined and the location of the manual traffic counting points. On each geographical sector average vehicle category distribution was assigned.

Temporal extrapolation of traffic profile, represented by FPj in equation (1), was obtained by mean data collected from continuous 24 hours traffic counting stations. Processing and analysis of this set of data made it possible to generate 24-hour traffic profiles normalized by morning on-peak values, for each one of the four types of day defined. Figure 7 shows an example of traffic profile obtained for sector 3, corresponding to a downtown zone of the city of Santiago. Two type of traffic profiles were used, one for buses and other for the rest of vehicles categories.

## MODEL RESULTS

The hot emission model developed is able to calculate hourly CO; THC; PM and NOx emissions on each arc of the traffic network for each vehicle category present in local vehicle fleet. As an example of this, Figure 8 shows a graphical representation of MP emissions from buses (kg/hour) in morning peak period of a working day on each arc of a certain street in a downtown zone of the city (Sector 3). This type of analysis can be done for each vehicle category, hour of the day and pollutant.

Another result of the model concerns total annual emissions for each sector of vehicle category and pollutant. As an example, NOx emissions from buses for nine sectors are graphically represented and compared to NOx emissions of the rest of vehicle categories (Figure 9). The major incidence of emissions from buses is in sectors 3 and 4, where great activity level of this vehicle category stands out. Additionally, it turns clear that total emissions, considering all vehicle categories, are greater in sectors 3; 5 and 6, which denote higher activity levels for older vehicles technologies, corresponding to lower social-economical sectors.

Emission scenario analysis permits emission projections from baseline emission inventory. This is normally used for effective evaluation of emission reduction strategies by mean emission estimation forecasting. The model is able to calculate predicted emissions using adequate projection of activity levels and emission factors. As an example of this, Figure 10 represents a comparison of 1997 base line and projected 2005 CO; PM; HC and NOx hourly emission at a micro-zone of sector 1. Emissions corresponding to 2005 were estimated by means of a projection of traffic flows from ESTRAUS model, which predicts a 65% total increase of the activity level (veh-km/year). In addition, increasing rates of 10% per year for the commercial vehicles fleet and 7% to 8% for the other vehicle categories were considered.  A rate of 2% of withdrawal per year for all vehicles categories was also estimated.

The type of results discussed above corresponds to micro-scale analysis, which constitutes a powerful tool for sector-scale emission management. Additional macro-scale analysis is also possible via integration of hourly street-by-street emission calculations of primary pollutants over the year and the entire traffic network. An example of this analysis is shown in Figure 11, where annual emission from desegregated vehicle categories and activity levels described by fuel consumption (FC) and vehicle-km/year (ACT) are shown, for the entire Metropolitan Region. This type of analysis allows identifying major primary pollutant emissions among desegregated vehicle categories. In the example of Figure 11, it is clear that more than 80% of total annual emissions of MP comes from buses and trucks, in spite of being lowest in annual mileage. Buses also have an important participation in NOx emission. On the other hand, private cars represent the highest contribution to CO and THC emission. This analysis can be improved by considering more desegregated vehicles categories, in order to allow more specific design strategies in terms of, say, emission regulation and transportation planning.

## DISCUSSION AND CONCLUSIONS

The hot emission model described above is able to render spatial and temporal high resolution emission distribution, thus allowing hourly emission estimates for street portions (arcs) defined on an real traffic network, for 15 vehicles categories.  Input data corresponds to real-time traffic data recollection and, in the case of gasoline light-duty vehicles, emission factors from an experimental program where local driving cycles were used.

The complexity of the methodology forces some hypothesis and simplifications to be improved in further research work. An example could be the development of an experimental program to determine emission factors for diesel vehicles as well as characterization of traffic flows at the urban road network in more detail and, thus, lifting the assumption of constancy along the day.

The methodology here discussed only concerns hot emission so it must to be extended to cold start and evaporative emissions as well. This implies the development of a methodology based on area-scale instead of arcs-scale as done here, which corresponds to the present work beeing done by the authors.

## ACKNOWLEDGMENTS

The financial support from the Environmental National Commission of Chilean Government (CONAMA) is greatly appreciated. The authors would like to acknowledge the help given by Dr. R. Acevedo and Dr. R. Tabensky in the revision of the manuscript.

REFERENCES

- Ntziachiristos L; Samaras Z; Eggleston S.; Gori N.; Hassel D.; Hickman A.J.; Joumard R.; Rijkeboer R.; Zierock K.H.. Computer Program to Calculate Emission from Road Transport. Copert III (Draft Report). European Environmental Agency. July, 1999. (http://vergina.eng.auth.gr/mech/lat/copert/copert.htm)

- Environmental Protection Agency. US-AP42. Supplement A to Compilation of Air Pollutant Emission Factors. Volume II: Mobile Sources, 1991. (http://www.epa.gov/orcdizux/models/)

- Zachariadis T; Samaras Z. Comparative Assessment of European Tools to Estimate Traffic Emissions. International Journal of Vehicle Design, 18, 312-325, 1997.

- Corvalán R.M.; Osses M.; Urrutia C.M.. Caracterización de las emisiones de vehículos livianos ciclo Otto en los flujos viales  de la Región Metropolitana – Chile. Revista Internacional Información Tecnológica. Vol 11, Nº3, May-Jun 2000

- Corvalán R.M.; Osses M. Estudio Experimental de Emisiones Asociadas a los Flujos Viales de la Región Metropolitana. 8º Congreso Chileno de ingeniería Mecánica. Vol II, 1109-1114, 1998.

- Corvalán R.M; Urrutia C.M. Emission Factors for Gasoline Light Duty Vehicles: Experimental Program in Santiago-Chile. Journal of the Air & Waste Management Association, 50, 2102-21111, December 2000.

- De Haan P., Keller M. Emission Factors for Passenger Cars: Application of Instantaneous Emission Modeling. 8º International Symposium “Transport and Air Pollution”, Graz, 1999.

- Bishop G.A.; Stedman D.H.. On-Road Carbon Monoxide emission measurement comparison for the 1988-1989 Colorado Oxy-Fuels Program. Environmental Science and Technology 24 (6) 843-847, 1990.

- Vanruymbeke C.; Guitton J.P.; Vindon R.; Pruvost C. Testing of an Alternative Method of Pollutant Emission Measurement for Passenger cars. Science of The Total Environment, 134 (1-3), 197-209, 1993.

- Cardelino C. Daily Variability of Motor Vehicle Emissions Derived from Traffic Counter Data, Journal of The Air & Waste Management Association. Vol 48, 637-645, 1998

- André M., Hammaström U. Driving speeds in Europe for pollutant emission estimation. 8º International Symposium “Transport and Air Pollution”, Graz, 1999.

- Zachariadis T., Samaras Z. An integrated modeling system for estimation of motor vehicle emissions. Journal of The Air & Waste Management Association. 49:1010-1026, September, 1999.

- Sturm P.; Almbauer R.; Sudy C.; Pucher K. Application of Computational Methods for the Determination of Traffic Emissions. Journal of The Air & Waste Management Association 47, 1204-1210, 1997.

- Don Young Kim, Jung Wk Kim. Development of a speciated, hourly and gridded air pollutants emission modeling system – A case study on the precursors of photochemical smog in Seoul metropolitan area, Korea. Journal of The Air & Waste Management Association. 50:340-347. March, 2000.

- Barth M.; An F.; Norbeck J.; Ross M. Modal Emission Modeling: A Physical Approach. Transportation Research Records, 50, 81-88, 1996.

- Loibl W., Orthofer R., Winiwarter, W. Spatially disaggregrated emission inventory for anthropogenic NMVOC in Austria. Atmospheric Environment 23, 37-48, 1993.

- Reynolds A.W.; Broderick B.M. (2000). Development of an Emission Inventory Model for Mobile Sources. Transportation Research Part D 5 (2000) 77-101.

- De Cea J., Fernández, J.E. Transit assignment for congested public systems: an equilibrium model. Transportation Science, 27 (2):133-147, 1993.

- Fernández J.E., De Cea J. An application of equilibrium modelling to urban transport planning in developing countries: the case of Santiago de Chile. Operational Research 90, Prergamon Press PLC, 1990.

- Watson H. Effects of Wide Range of Drive Cycles on the Emission from Vehicles of Three Levels of Technology. SAE Paper Nº 950221, 1995.

- Shayler P.J., Dartnton N.J. and Ma T. Factors Influencing Drive Cycle Emissions and Fuel Consumption. SAE Paper Nº 971003, 1997.

- Joumard R., André M., Vidon R., Tassel P. and Pruvost C. Influence of Driving Cycles on Unit Emissions from Passenger Cars. 8º International Symposium Transportation and Air Pollution. Graz, 1999.

- Ntziachristos L., Samaras Z. Speed Dependent Representative Emission Factors of Catalyst Passenger Cars and Influencing Parameters. 8º International Symposium Transportation and Air Pollution. Graz, 1999.

- Ntziachiristos L and Samaras Z. COPERT II Computer Programme to Calculate Emission from Road Traffic. European Environmental Agency. 1997. (http://vergina.eng.auth.gr/mech/lat/copert/copert.htm)

Table 1. Vehicles Categories

| VEHICLE TYPE | CATEGORY | DESCRIPTION |
| --- | --- | --- |
| BUSES | BUS A | Urban Buses pre-EPA-91 |
|  | BUS B | Urban Buses EPA-91 |
|  | BUS C | Urban Buses EPA-94 |
|  | BUS D | Other Buses |
| TRUCKS | TRUCK A | Light and medium-duty trucks |
|  | TRUCK B | Heavy-duty truck |
| PRIVATE VEHICLES | PPV-CAT | Light Duty catalytic private passenger cars |
|  | PPV-NCAT | Light Duty non-catalytic private passenger cars |
| COMMERCIAL VEHICLES | CV-CAT | Light and Medium Duty catalytic commercial vehicles |
|  | CV-NCAT | Light and Medium Duty non-catalytic commercial vehicles |
|  | CVD | Light and Medium Duty diesel commercial vehicles |
| TAXIS | T-CAT | Catalytic taxis |
|  | T-NCAT | Non-catalytic taxis |
| MOTORCYCLES | M-2S | Two stroke motorcycles |
|  | M-4S | Four stroke motorcycles |

- Table 2. Characteristics of the nine local driving cycles

|  | AVERAGE | MAX SPEED | DURATION | ACCELARATION | MAX | IDLE |
| --- | --- | --- | --- | --- | --- | --- |
| CYCLE | SPEED |  |  | TIME | ACCELERATION | TIME |
|  | km/h | km/h | sec | % | m/s2 | % |
| STD 05 | 3.0 | 25.2 | 239.0 | 14.2 | 2.0 | 67.1 |
| STD 10 | 11.1 | 36.0 | 239.0 | 32.5 | 1.5 | 25.4 |
| STD 20 | 19.6 | 57.6 | 249.0 | 32.4 | 2.0 | 25.6 |
| STD 30 | 29.7 | 61.9 | 244.0 | 42.9 | 2.1 | 14.3 |
| STD 40 | 38.4 | 72.1 | 249.0 | 48.8 | 1.8 | 6.4 |
| STD 50 | 46.3 | 73.3 | 274.0 | 42.2 | 2.3 | 4.0 |
| STD 60 | 55.3 | 77.1 | 274.0 | 49.1 | 3.5 | 1.5 |
| STD 70 | 66.4 | 84.7 | 274.0 | 38.2 | 2.3 | 0.7 |
| STD 80 | 72.9 | 94.8 | 279.0 | 43.6 | 2.8 | 1.8 |

- Table 3. Emission factors for Diesel vehicles ( COPERT II 24)

| VEHICLE CATEGORY | POLLUTANT | EMISSION FACTOR (g/km) | ORIGINAL SOURCE APPLICATION |
| --- | --- | --- | --- |
|  | MP | 0.0000125 V2 – 0.000577 V + 0.2880 |  |
| CVD | CO | 0.00020 V2 – 0.0256 V + 1.8281 | Light-duty diesel |
|  | THC | 0.000066 V2 – 0.0113 V + 0.6024 | vehicles |
|  | NOx | 0.00014 V2 – 0.01592 V + 1.4921 | < 3.5 Ton |
|  | MP | 12.09253 V-0.7360 |  |
| BUS A and D | CO | 59.003 V -0.7447 | Conventional |
|  | THC | 43.647 V -1.0301 | Buses |
|  | NOx | 89.174 V -0.5185 |  |
|  | MP | 5.109585 V -0.7360 |  |
| BUS B | CO | 29.5015 V -0.7447 | Urban Buses |
|  | THC | 32.73525 V -1.0301 | Stage I |
|  | NOx | 62.4218 V -0.5185 | (91/542/EEC) |
|  | MP | 3.14436 V -0.7360 |  |
| BUS C | CO | 23.6012 V -0.7447 | Urban Buses |
|  | THC | 30.5529 V -1.0301 | Stage II |
|  | NOx | 44.587 V -0.5185 | (91/542/EEC) |
|  | MP | 4.2074 V –0.7174 |  |
| TRUCK A | CO | 18.64 V –0.6945 | Light and Medium- |
|  | THC | 30.09 V –0.8774 | duty truck |
|  | NOx | 46.43 V –0.7535 (0 to 60 km/h) 0.00077 V2 – 0.10045 V + 5.346 (60<V<100 km/h) |  |
|  | MP | 8.712V -0.7105 |  |
| TRUCK B | CO | 29.824 V -0.6945 | Conventional |
|  | THC | 32.096 V -0.8774 | heavy-duty |
|  | NOx | 86.688 V -0.6061 | trucks |
|  | CO | -0.001 V2 + 0.172 V + 18.1 | Conventional |
| M-2S | THC | 0.0035 V2 - 0.409 V + 20.1 | heavy-duty |
|  | NOx | 0.00003 V2 - 0.002 V + 0.064 | trucks |
|  | CO | 0.0123 V2 - 1.19 V + 42.8 | Conventional |
| M-4S | THC | 0.0022 V2 - 0.257 V + 9.28 | heavy-duty |
|  | NOx | 0.00005 V2 - 0.0008 V + 0.1 | trucks |

Table 4. Emission factors evaluated at 20 km/h average speed.

| VEHICLE TYPE | #### LOCAL EMISSION FACTORS |  |  | #### COPERT II EMISSION FACTORS24 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO | THC | NOx | CO | THC | NOx | PM |
| PRIVATE-CAT | 2.10 | 0.11 | 0.32 | 2.50 | 0.17 | 0.42 | - |
| COMMERCIAL-CAT | 0.55 | 0.35 | 0.60 | 1.51 | 0.13 | 0.40 | - |
| PRIVATE-NO CAT | 35.47 | 2.10 | 2.40 | 32.12 | 3.03 | 1.76 | - |
| COMMERCIAL -NO CAT | 39.92 | 3.05 | 3.64 | 31.94 | 3.40 | 1.82 | - |
| COMMERCIAL-DIESEL | na | na | na | 1.40 | 0.40 | 1.23 | 0.28 |
| BUS A AND D | na | na | na | 6.34 | 1.99 | 18.86 | 1.33 |
| BUS B | na | na | na | 3.17 | 1.50 | 13.21 | 0.56 |
| BUS C | na | na | na | 2.54 | 1.40 | 9.43 | 0.35 |
| TRUCK A | na | na | na | 1.28 | 2.17 | 4.86 | 0.49 |
| TRUCK B | na | na | na | 2.05 | 2.32 | 7.75 | 1.04 |
| M-2S | na | na | na | 21.14 | 13.32 | 0.076 | - |
| M-4S | na | na | na | 23.92 | 5.02 | 0.104 | - |

na: not available local emission factors for these vehicle categories

Figure 1. Traffic network of Metropolitan Region of Santiago, Chile

DRIVING CYCLES EXPERIMENTAL PROGRAM

# STDv Driving Cycles

EMISSION FACTORS EXPERIMENTAL PROGRAM

EF i,k(v)

TRAFFIC FLOWS CHARACTERIZATION EXPERIMENTAL PROGRAM

FP j,k; C j,k

HOT EMISSION MODEL

E h,i,j,k

TRANSPORTATION MODEL

(ESTRAUS)

# Fj; vj; Lj

Figure 2. Hot Emission Model Scheme.

Figure 3. Example of speed-time traces obtained from the instrumented car

Figure 4. Driving cycle of 30 km/h average speed

Figure 5. Daily flow distributions in sector 3 (downtown): (a) private vehicles and taxis; (b) commercial vehicles, buses and trucks.

Figure 6. Sectors with homogeneous vehicle category distribution and location of manual counting stations

Figure 7. Example of traffic flow profile

Figure 8. Example of street by street hourly emission estimation for vehicle category and pollutant predefined: PM Buses emission (kg/h), in morning on-peak hour atl downtown zone (Sector 3).

Figure 9. Total annual emission by geographical sectors: NOx Buses emission compared with NOx emission from the rest of vehicles categories.

Figure 10. Comparison of 1997 and 2005 CO; PM; HC and NOx hourly emission by arc at a micro-zone of sector 1.

Figure 11. Annual emission, activity level (ACT) and fuel consumption (FC) distributions per vehicle categories