A model to determine primary air pollutant emissions from urban buses in Santiago de Chile

M. Osses1, C. Gherardelli1, F. Fresard2

1Department of Mechanical Engineering, University of Chile, Chile

2DICTUC, Pontificia Universidad Católica, Chile

Abstract

A methodology for estimating emissions from urban buses using passive traffic data is described. A software simulation of the physical road network and traffic activity is linked to a set of emission factors for estimating emissions of particulate matter (PM), carbon monoxide (CO), unburned hydrocarbons (HC), oxides of nitrogen (NOX), sulphur dioxide (SO2), methane (CH4), nitrous oxide (N2O), ammonia (NH3) and fuel consumption (FC). Observed traffic data is compared to simulated data from a travel demand model, on the metropolitan area of Santiago de Chile.

Urban buses are one of the largest sources of primary particulate matter and nitrogen oxides in Santiago. Total fleet reaches 8469 units, divided into three types according to their emission factors. Conventional buses represent 41% of the fleet, accounting 66% of particulate matter and nitrogen oxides. On the other hand, EURO II buses represent 25% of the total fleet with only 8% of the same pollutant emissions per year. An accurate prediction of both traffic data and emissions allows to better evaluating abatement strategies to decrease the ambient concentrations of a wide range of pollutants.

1  Introduction

Modelling atmospheric pollutant emissions from traffic activity data has been widely used for developing emission inventories in urban areas [1,2]. There are different approaches to obtain spatially desegregated traffic information such as surveys [3], transportation models [4] or real-time data [5]. Traffic surveys have the major limitation that they supply data limited to a certain time and location, whereas real-time traffic data require an adaptive urban traffic control system. Detailed and complex transportation models can simulate entire urban domains, but they still do not contain all the information needed for the compilation of a complete inventory of emissions [5]. Particularly, public transport activity (e.g. traffic volume and mean velocities) are not always fully represented by strategic transportation models [6]. Public transportation is a dominant source of urban air pollution; therefore it is necessary to quantify emission levels as accurately as possible. The more precise the description of the driving behavior and fleet composition, the more reliable the emission estimate will be [7].

Urban areas with high population densities are often exposed to poor air quality. Several recommendations for developing countries to address the problems of urbanization and air pollution have been proposed [8], such as improvement of monitoring and emissions inventory capabilities in cities. These are prerequisites for preventive measures to be introduced. Proper urban and transportation planning can achieve significant improvements in air quality. The introduction of clean technologies is a major goal in air pollution management. This paper describes the results of a project that estimates on-road emissions for Santiago de Chile, where public transportation is a major source of particulate matter and nitrogen oxides urban emissions.

2  Emission calculation methodology

The determination of emissions from urban buses is based upon the use of emission factors, which relate the mass of pollutant emission to speed and distance covered over a certain trip. Since no emission factors for buses are available for Chile, the values from the European computer program COPERT II [9] have been used in this study, where some of then have been adjusted to local conditions (see Table 1). Adjustments have been made taking into consideration diesel fuel quality and engine technologies available in Santiago.

There are three main urban bus categories in Santiago, classified according to their emission level. Category 1 represents conventional buses, reaching 3429 vehicles (41% of the total urban bus fleet used for public transportation). Category 2 comprises urban buses complying with either EURO 1 or EPA 1991 emission standards, sharing a 35% of the fleet with 2945 units. Finally, category 3 represents those buses equipped with emission control technology according to either EURO 2 or EPA 1994 standards, with 2095 units (25% of the fleet). The emission factors derived for each category are as follows:

Table 1. Emission factors for urban bus categories

(v: vehicle speed in km/h)

| Bus category | Component | Emission factor (g/km) |
| --- | --- | --- |
| Category 1 Conventional | PM† CO HC NOx FC† | 12.09253·v(-0.7360) 59.003·v(-0.7447) 43.647·v(-1.0301) 89.174·v(-0.5185) 864.1·v(-0.4318) |
| Category 2 EURO 1 | PM CO HC NOx FC† | 5.109585·v(-0.736) 29.5015·v(-0.7447) 32.73525·v(-1.0301) 62.4218·v(-0.5185) 1001.3·v(-0.4318) |
| Category 3 EURO 2 | PM CO HC NOx FC† | 3.14436·v(-0.736) 23.6012·v(-0.7447) 30.5529·v(-1.0301) 44.587·v(-0.5185) 1645.9·v(-0.4318) |

† These values have been adjusted from COPERT II to local conditions

Emission factors for unregulated gases are considered to be constants, without difference among categories. Thus, CH4: 0.175 (g/km), N2O: 0.03 (g/km) and NH3: 0.003 (g/km).

The emission rate per unit of time is calculated as follows:

Eijktv=TTVjt⋅Lj⋅EFikv⋅Fjk			(1)

where

Eijktv:	Emission rate (g/unit-time) of a particular pollutant i driven on arc j by vehicles of category k, during a period of time t at an average speed v

TTVjt:Total traffic volume (veh/unit-time) driven on arc j during a period of time t

Lj:	Length (km) of the arc j

EFikv:	Emission factor (g/km) for the pollutant i, representative for vehicle category k at an average speed v

Fjk:	Fraction over total vehicle counts of vehicle category j driven on arc k

The transportation model used in this study is ESTRAUS, with an urban road network comprising 7567 arcs distributed over the greater Santiago area, which is divided into 264 zones. As with most transportation models, ESTRAUS is built on four main stages: trip generation/attraction, trip distribution, modal choice/split, and trip assignment. The last three stages are solved through a simultaneous equilibrium, generating a solution that reflects a dynamic interaction among them, instead of a sequential modeling approach. This model is used for strategic planning and covers two main periods of time: morning peak hour (typically 07:30-8:30 hrs) and the inter-peak hour (typically 10:30-11:30 hrs), both timings corresponding to a weekday.

Regarding emission calculation, the main outputs from ESTRAUS are traffic volume and time needed to drive over a given arc of the urban road network. Traffic volume is divided into both fix and variable flow. Fix flow represents public transportation (urban buses) and it is received as input by the model, but it is modified during certain stages of modeling, especially at peak-time hours. On the other hand, variable flow includes private transportation modes and it is associated to variable timing, which allows calculating the average speed while driving over a given arc. Figure 1 shows the urban road network for Santiago.

Figure 1. Urban road network

Urban bus velocities are inferred from variable timing assigned to private transportation, under the assumption that buses are directly affected by private car activity level when driving or competing on the same arc. The transportation model assumes six different correction factors according to the arc type, ranging from 1.11 to 1.55. However, when comparing results using these factors with traffic information obtained through surveys or traffic control systems, the modeled data presents major differences. At morning peak-hour there is an overall 6% overestimate comparing the transportation model with survey data, reaching up to 12% on the main road crossing Santiago’s city center. Speed variability is also greater for the model, due to its strong correlation with variable timing or private transport velocity. Figure 2 illustrates the differences between urban bus velocities at morning peak hour along Alameda Av., in the heart of Santiago City, which is characterized by high congestion and an important public transport participation in total traffic volume. The same Figure shows both PM and NOx emissions on the same Avenue (the higher the speed the lower the emissions).

Figure 2:	Urban bus velocities on Alameda Av. at morning peak hour (top), particulate matter in kg/hr (center) and NOx emissions in kg/hr (bottom).

A calculation of emissions over Santiago’s city center (1472 arcs) at morning peak hour shows that 2.3% difference on bus velocity produces 16% difference in PM estimates and 9% for NOx (comparison between ESTRAUS model and survey data). These differences have an important impact on total emissions due to high traffic activity concentrated in the center urban area. Total bus activity during a weekday is shown in Figures 3 and 4.

Figure 3: Geographical and daily distribution of urban buses traffic volume.

Figure 4: Diurnal variation of measured traffic volume for a weekday.

3  Results

Emission Inventory from mobile sources for Santiago de Chile has been calculated for years 1997, 2000 and 2005, in accordance with the methodology described earlier. Figure 5 shows the results obtained in 1997, using urban bus velocities modeled by ESTRAUS (i.e. assuming that urban bus velocities are a fraction of equilibrium velocity per each arc of the urban network). Both pollutant emissions and fuel consumption (FC) are given in thousand tons per year, whereas activity level (ACT) is calculated in vehicle·kilometer/year, and passenger capacity (PAS) represents carried passenger·kilometer/year.

Figure 5:	Distribution per vehicle type for total emissions, fuel consumption and activity level from mobile sources in Santiago, 1997. PM: particulate matter (PM10), SO2: sulfur dioxide, NOx: nitrogen oxides, CH4: methane, HC: total unburned hydrocarbons, N2O: nitrous oxide, CO: carbon monoxide, NH3: ammonia, FC: fuel consumption, ACT: vehicle activity level (103·veh·km/year), PAS: passenger capacity (103·passenger·km/year).

Urban buses are an important source of particulate matter, SO2 and NOx (34-40% of total mobile sources), burning a 20% of the fuel used for transportation in the region, sharing 5% of total traffic activity and carrying 55% of total number of passengers driven by on-road vehicles.

As an example, PM emissions from urban buses have been calculated for nine zones covering whole Santiago’s urban area. Compared with total mobile sources hot emissions, public transportation produces 43% PM. Its effect is more important in the center and north-east areas, as it is shown in Figure 7.

30%

58%

61%

59%

19%

51%

38%

35%

37%

Figure 6:	Annual PM emissions from urban buses compared with those from other on-road vehicles by geographical zones in Santiago.

4  Conclusions

Pollution from road traffic plays an important role in urban areas, for this reason it is important to quantify the emissions levels as accurate as possible. Urban buses are the major contributors of particulate matter and nitrogen oxides from on-road mobile sources in Santiago, therefore it is necessary to generate good traffic activity in order to adequately feed modal emission models based upon total traffic volume and vehicle speed.

Urban bus velocities are an important parameter for estimating total emissions from this vehicle category. Survey data allows accurate estimations but they are limited to a certain time and location. Specific models for urban driving behavior representative of public transportation would be an important contribution to better evaluate emissions in megacities for developing countries. Also, modeling allows predicting the effect of measures for reducing pollutant emissions.

Nine thousand buses are responsible for 40% of PM emissions from all mobile sources in Santiago, where total vehicle fleet reaches 780.000 on-road vehicles. This participation arises up to 60% in certain areas with high traffic activity. Particulate matter emission estimates can change in 16% due to 2% speed variability from urban bus activity (9% for NOx).

References

[1]	Zachariadis, T., Samaras, Z. Comparative assessment of European tools to estimate traffic emissions. International Journal of Vehicle Design, 18, pp. 312-325, 1997.

[2]	Barth, M., An, F., Norbeck, J., Ross, M. Modal emissions modelling: A physical approach. Transportation Research Record, 50, pp. 81-88, 1996.

[3]	Cardelino, C. Daily variability of motor vehicle emissions derived from traffic counter data. Journal of the Air & Waste Management Association, 48(7), pp. 637-645, 1998.

[4]	Algers, S., Bernauer, E., Boero, M., Breheret, L, Di Taranto, C., Dougherty, M. Review of Micro-Simulation Models. Institute of Transport Studies, Leeds, UK, 1998.

[5]	Reynolds, A.W., Broderick, B.M. Development of an emission inventory model for mobile sources. Transportation Research, 5D(2), pp. 77-101, 2000.

[6]	Bull, M.A., Zimmann, R. Traffic emission data for air quality reviews. Traffic Engineering + Control, 9, pp. 470-472, 1997.

[7]	Sturm, P.J., Almbauer, R., Sudy, C., Pucher, K. Application of computational methods for the determination of traffic emissions. Journal of the Air & Waste Management Association, 47, pp. 1204-1210, 1997.

[8]	Mage, D., Ozolins, G., Peterson, P., Webster, A., Orthofer, R., Vandeweerd, V., Gwynne, M. Urban air pollution in megacities of the world. Atmospheric Environment, 30(5), pp. 681-686, 1996.

[9]	COPERT II, Computer Program to Calculate Emissions from Road Transport. Methodology and Emissions Factors User Manual, 2nd ed. European Environment Agency & European Topic Center on Air Emission, 1997.